# First-Order Digital Delta-Sigma Modulator

Vivado Behavioral Simulation — Setup, RTL, Testbench and Verification

Project notes | Version 1.0 | Simulation-only milestone

# 1. Objective

Build and simulate a simple first-order digital delta-sigma modulator (DSM) using Verilog in AMD/Xilinx Vivado. Verify accumulator and carry-bit behavior in a waveform before moving to the more complex MASH 1-1 architecture.

No physical FPGA board is required for this exercise.

Synthesis, implementation, and bitstream generation are not required for behavioral simulation.

Python is optional at this stage; it will be useful later for longer output sequences and spectral analysis.

# 2. Short theory

A first-order DSM can be modeled as an accumulator that repeatedly adds a fixed input K. The accumulator keeps the lower W bits of the sum, while the carry-out is used as the one-bit output y.

For accumulator width W, the fractional input represented by K is:  x = K / 2^W

At each active rising clock edge:

S[n] = A[n] + K

y[n] = floor(S[n] / 2^W)  (carry-out)

A[n+1] = S[n] mod 2^W  (lower W bits)

For this exercise, W = 4 and K = 4, so x = 4/16 = 0.25. Starting from accumulator value zero, the accumulator cycles through 0, 4, 8, 12, 0, ... and the output repeats 0, 0, 0, 1, ... . The average over a complete four-cycle pattern is 1/4 = 0.25.

This verifies fractional-average behavior. It does not, by itself, establish the full spectral noise-shaping performance of a complete MASH design.

# 3. Tools and files

AMD/Xilinx Vivado with its behavioral simulator (XSim).

RTL source: first_order_dsm.v

Simulation testbench: tb_first_order_dsm.v

The design source describes the circuit. The testbench is simulation-only: it generates the clock, applies reset, sets K, and ends the simulation. Set tb_first_order_dsm as the simulation top.

# 4. RTL design: first_order_dsm.v

module first_order_dsm #(
    parameter W = 4
)(
    input  wire         clk,
    input  wire         rst,
    input  wire [W-1:0] K,
 
    output reg          y,
    output reg  [W-1:0] acc
);
 
    wire [W:0] sum;
 
    // Add the input to the current accumulator.
    // The extra bit preserves the carry-out.
    assign sum = {1'b0, acc} + {1'b0, K};
 
    always @(posedge clk) begin
        if (rst) begin
            acc <= 0;
            y   <= 0;
        end
        else begin
            acc <= sum[W-1:0];
            y   <= sum[W];
        end
    end
 
endmodule

Important detail: sum is W+1 bits wide. The extra most-significant bit stores the carry-out; without it, the carry could be lost.

# 5. Testbench: tb_first_order_dsm.v

`timescale 1ns / 1ps
 
module tb_first_order_dsm;
 
    reg clk;
    reg rst;
    reg [3:0] K;
 
    wire y;
    wire [3:0] acc;
 
    // Instantiate the design under test.
    first_order_dsm #(
        .W(4)
    ) dut (
        .clk(clk),
        .rst(rst),
        .K(K),
        .y(y),
        .acc(acc)
    );
 
    // Generate a clock with a 10 ns period.
    initial clk = 0;
    always #5 clk = ~clk;
 
    // Apply reset and run the simulation.
    initial begin
        rst = 1;
        K   = 4'd4;
 
        repeat (2) @(posedge clk);
 
        // Release reset just after a rising edge.
        #1 rst = 0;
 
        // Observe 20 more rising clock edges.
        repeat (20) @(posedge clk);
 
        #1 $finish;
    end
 
endmodule

The testbench creates a 10 ns clock (toggle every 5 ns), holds reset high for two rising edges, then deasserts reset 1 ns after a rising edge. This avoids changing reset at the same simulation instant as the design's clocked logic. The simulation ends after 20 additional rising edges.

# 6. Step-by-step Vivado setup

Create the project: Open Vivado → Create Project → choose a project name and folder → select RTL Project. You may leave 'Do not specify sources at this time' checked and add the files afterward.

Choose a part/board if prompted: For simulation-only work, no physical board is needed. If you know the intended PYNQ board, choose its matching board/part. Otherwise select a compatible installed part; do not guess a board for later hardware implementation.

Add the RTL design: Flow Navigator → Add Sources → Add or Create Design Sources → Create File → Verilog → name it first_order_dsm. Paste the RTL above and save.

Add the testbench: Add Sources → Add or Create Simulation Sources → Create File → Verilog → name it tb_first_order_dsm. Paste the testbench above and save.

Check the hierarchy: Under Design Sources, confirm first_order_dsm.v. Under Simulation Sources, confirm tb_first_order_dsm.v.

Set the simulation top: Under Simulation Sources, right-click tb_first_order_dsm and select Set as Top if it is not already the top.

Launch simulation: Flow Navigator → Run Simulation → Run Behavioral Simulation. Vivado compiles the design and opens the simulator.

Add signals to the waveform: If needed, select tb_first_order_dsm in Scopes. In Objects, select clk, rst, K, acc, and y, then right-click and choose Add to Wave Window.

Run the test: Use Run All. Because the testbench calls $finish, simulation should stop automatically. Alternatively, run for a sufficient duration, such as 250 ns.

Fit the display: Use Zoom Fit so the available simulation time fits in the waveform window. Change acc's display radix to Unsigned Decimal or Hexadecimal if helpful.

# 7. Expected behavior and verification

After reset is deasserted, the first few active rising edges should follow this table:

| Edge | Previous acc | Sum (acc + 4) | New acc | y (carry) |
| --- | --- | --- | --- | --- |
| 1 | 0 | 4 | 4 | 0 |
| 2 | 4 | 8 | 8 | 0 |
| 3 | 8 | 12 | 12 | 0 |
| 4 | 12 | 16 | 0 | 1 |
| 5 | 0 | 4 | 4 | 0 |
| 6 | 4 | 8 | 8 | 0 |
| 7 | 8 | 12 | 12 | 0 |
| 8 | 12 | 16 | 0 | 1 |

Expected repeating accumulator sequence: 0 → 4 → 8 → 12 → 0 → ...

Expected repeating output sequence: 0, 0, 0, 1, 0, 0, 0, 1, ...

There is one '1' in every four samples, so the average over complete repeating groups is 0.25. Reset cycles are not part of this repeating pattern.

# 8. Troubleshooting

No waveform appears — Add clk, rst, K, acc, and y from the Objects panel to the Wave window, then run the simulation.

Simulation time does not advance — Use Run All or Run For. Check that the testbench is set as the simulation top.

Syntax or compile error — Read the first error in the Tcl Console/Messages panel. Check semicolons, module names, and that the full code is in the correct file.

Unknown module — Confirm first_order_dsm.v is under Design Sources and the instantiated module name matches exactly.

Unexpected values — Confirm K is 4'd4, reset is initially high, and the testbench instantiates W=4.

acc displays in binary — Right-click acc in the waveform and change Radix to Unsigned Decimal or Hexadecimal.

# 9. What is not needed yet?

Synthesis is not required to check RTL behavior in this first simulation.

Implementation and bitstream generation are not required until a physical FPGA test is planned.

The PYNQ board is not required for the current milestone.

Python is not required to generate this waveform; it will be introduced later for automated analysis, average checks, plots, and FFT/PSD analysis.

# 10. Suggested GitHub repository structure

first-order-dsm/
├── README.md
├── docs/
│   └── First_Order_DSM_Vivado_Simulation_Guide.docx
├── rtl/
│   └── first_order_dsm.v
├── sim/
│   └── tb_first_order_dsm.v
└── screenshots/
    └── first_order_dsm_waveform.png

After successful simulation, save a waveform screenshot in screenshots/. In README.md, record the aim, parameters (W=4, K=4), expected sequences, Vivado version, and actual observed result. Do not claim the test passed until you have run it.

# 11. Next steps after this milestone

Run the testbench and confirm the expected accumulator and carry sequence.

Try K values such as 1, 4, 8, and 12; check that the long-term output average approaches K/16. K=0 should produce all zeros; K=15 should average 15/16.

Extend the testbench to count output ones over a fixed number of samples and compare the measured average against K/16.

Export or record a longer output sequence and use Python/NumPy to plot the bitstream and calculate its average.

Study first-order quantization-error behavior, then implement the second first-order stage needed for MASH 1-1.

Build the MASH 1-1 cancellation logic and verify it against a reference model before considering FPGA implementation.

Only after RTL simulation is reliable, decide whether to synthesize and test on the available PYNQ board.

# 12. Completion checklist

☐ Project opens successfully in Vivado.

☐ RTL and testbench files are saved in the correct source categories.

☐ Behavioral simulation compiles without errors.

☐ Waveform contains clk, rst, K, acc, and y.

☐ Accumulator cycles through 0, 4, 8, 12, 0 after reset.

☐ Output repeats 0, 0, 0, 1.

☐ Waveform screenshot is saved for the repository.

☐ README records the actual Vivado version and observed result.